MIG Welding 101

A lot of people will have access or even own a Mig welder, usually a small hobby units from the likes of Machine Mart.

How a Mig Welder Works

To understand how to get the best from a Mig welder I find it best if you first understand how it works.

In its most basic form a Mig welder works by fusing the metal together within an inert gas shield with the use of electricity.

A current is passed through a filler wire via the contact tip.  When the wire strikes the earthed job piece it completes a circuit, the resulting arc melts both the job piece and the filler wire creating a weld pool. The welding torch is moved along the seam melting and filling as it goes.

The Gas is used to prevent the molten pool from burning and causing bubbles and weakness in the weld, it does this by creating a shroud around the pool and excluding the oxygen.

Safety first

Before starting any welding you must have the proper safety equipment.

Welding mask. You must use a mask because of the high strength UV light produced by the arc. If you weld without a mask it will burn the skin off the inside of your eyes. You have been warned.

The most important part of the mask is the welding glass shade; it should be clean and not cracked. I generally use a shade 8 if I’m only doing a little welding because it gives better visibility but it doesn’t give enough protection if your going to be welding for a while. If I’m welding for a long time I’d use a shade 10.

Welding gloves/gauntlets. To protect you hands not only from the heat but also from weld splatter (bits of molten metal thrown off during welding) and the powerful UV light as it will burn the skin on your body as well.

Overalls. Preferably fire retardant. As with the gloves, to protect from weld splatter and UV light

Gas vs. No Gas

Gas refers to the bottle of inert gas used to produce the gas shroud.  Most people use a CO2 gas because it’s cheaper and readily available. It does do the job but I personally don’t like using it because I think it causes too much splatter, I prefer to use an Argon mix, it is more expensive but I feel it gives a cleaner nicer weld.

No Gas refers to a Flux cored wire which creates a gas shroud by melting the flux when the arc is struck.

This has its advantages; you don’t need to mess on with a bottle and getting refills, it’s easier to use outside because the shroud isn’t as affected by the wind.

And its disadvantages; messy weld, a lot of weld splatter, and expensive wire

Setting up the Mig welder

I’ve found most people don’t get the best from their welders because they aren’t set up correctly

Setting the power

If you take for example the welder in the picture above, It has a Max power rating of  150 amp and three settings selectable by the dial switch and a min and a max or each setting giving it a total of six possible power settings  increasing by 25 amps a time

1 min = 25 amp

1 max = 50 amp

2 min = 75 amp

2 max = 100 amp

3 min = 125 amp

3 max = 150 amp

Now the general rule of thumb is you need 1 amp to weld every 0.01” of steel

So to weld 18 gauge steel (0.49”) you need 50 amp or setting 1 max on the example welder.

| 14 Gauge | .083 “ | 2.108 mm |
| --- | --- | --- |
| 15 Gauge | .072 “ | 1.829 mm |
| 16 Gauge | .065 “ | 1.651 mm |
| 17 Gauge | .058 “ | 1.473 mm |
| 18 Gauge | .049 “ | 1.245 mm |
| 19 Gauge | .042 “ | 1.067 mm |
| 20 Gauge | .035 “ | .889 mm |

As you can see it isn’t always possible to get the correct power setting for the gauge of steel you’re welding in which case it’s better to take the higher setting to ensure a good weld.

Setting the wire speed

Once you have the power setting right you need to set the wire speed. If you start with your wire speed high on a test piece, when you pull the trigger and strike the arc you will probably feel the wire hitting the job and the torch will buck and be difficult to control. Slowly turn down the wire speed until you hear a sound like sizzling bacon, if you turn it down to much the wire will strike the arc and then disappear back up the torch.

Basic technique

Once you welder is set up you’re ready to start welding.

If you’re Welding on the car make sure you have disconnected the battery, you’re going to be passing a lot of electricity though the car and this can damage sensitive electronics, it can also fry the battery. Also be aware you are going to be producing a lot of heat and sparks, so any carpets, interiors, glass, Flammable liquids should be kept as far away as possible and covered. I always keep a washing up liquid bottle filled with water next to me when welding to extinguish possible fires.

Ensure the job you’re working on is as clean as possible, free from any paint, rust, or contaminants. Remember that in order to weld you need to make a good electrical circuit

Connect the earth strap to the job, this needs to make a good electrical contact so it must be on a nice clean section of the job.

Turn on the power to the machine, and turn on the gas (if your using it), make sure the gas is coming through the torch when you pull the trigger, it should be a light hiss (adjust the regulator if necessary).

Hold the torch approx between 10 and 15 mm from the job at a slight angle and lower your mask.

When you pull the trigger you will se the arc strike and the weld pool will start to form.

Keep a close eye on the weld pool once it has reached a size of about 5 to 7 mm start to move the torch slowly towards you, if it starts to get to big either move the torch faster or release the trigger allowing the job to cool before starting again.

Always move the welding torch towards you so you can see what you’re doing

Practice makes Perfect.