Small Dent Removal

Tools and Materials required

Heat gun or hair dryer

Round handled tool; you will need something with a large rounded top made from either hard plastic or wood. I generally use a screwdriver, the handle needs to have no sharp or pointed edges that could cause high spots in the repair.

Miscellaneous; any other tools required for removing interior trim etc.

Rubbing out dents

Very small or shallow dents like the ones caused by car doors bumping in the car park can often be removed with out breaking the paint surface (so no need to respray).

Although this technique isn’t guaranteed to work it is worth trying first before you go in with grinders blazing.

Firstly start by removing any bits of trim that prevent access to the other side of the panel, so if the dent you are working on is on the rear quarter then you will need to remove any interior bits of trim and speakers. With all the trims removed check you can get your hand on to the back of the dent, if you can touch it with your fingers you stand a good chance of rubbing it out.

Give the panel you are working on a quick wash and dry.

Gently heat the dent with a heat gun, be very careful to heat it up gently because you want to avoid burning the paint and causing it to bubble.

With the dent warmed take a round handled screwdriver or similar tool and starting with a gentle pressure begin to rub on the back of the dent from behind. Looking from the painted side you should see where you are applying the pressure as you are rubbing; you will need to adjust the pressure applied and the location of the rubbing as you go.

Don’t apply to much pressure in one place or it will cause high spots

Try to keep the area as warm as possible this will help to make the metal a bit more malleable.

Keeping a close eye on the outside of the panel you should start to see the dent disappearing while rubbing.

When you are happy you have got the dent out as much as possible cool the panel with some cold water on a cloth then you can clean the area and refit the interior trims.

It would be almost impossible for me to describe the correct way to rub the dent as each dent can be different depending on how and from what angle it was made and the curve of the panel it was made in.

You do want to avoid creating any high spots or creases in the repair area. If you do get high spots or you are not happy with the way the dent has come out then the dent will need to be filled and re painted.